Dear __.

We are facing problem update CYPD3175 FW via Power Board

We selected CCG3PA Power Adapter with Direct Feedback Control example from Power SDK

We used PSoC creator 4.2.

# Custom board base on CYPD3175

# Power board CY4532

# Using the CY4532 Power Board as a Programmer (base on EZ-PD™ CCG3PA Evaluation Kit Guide.pdf)

# Bootloader version in above example

## Path bootloader in power SDK:

- C:\Program Files (x86)\Cypress\EZ-PD CCGx Power SDK\CCGx\Firmware\projects\CYPD3175-24LQXQ_pa_direct_fb\CYPD3175-24LQXQ_pa_direct_fb.cydsn\Bootloader

- .\Bootloader\CYPD3175-24LQXQ_cc_src_boot_3_1_2_1484_0_0_0_pa.hex

- .\Bootloader\CYPD3175-24LQXQ_cc_src_boot_3_1_2_1484_0_0_0_pa.elf

## See Appendix A1 , Appendix A2

## Bootloader version of cypd3175 (first connection via power board) we read device via Cypress EZ-PD™ Configuration Utility) –see Appendix A3

## Bootloader parameters and build-in configuration in PSoC IDE

### Parameters –Appendix A5

### Build-in –Appendix A6

## Boards, Cables, and Accessories Needed

The following items are needed to use the CY4532 Power Board as a Programmer:

- Items provided with the kit:

  - CY4532 Power Board

  - 12 V 50.4 W DC power adapter

- Items not provided with the kit:

  - Hardware with Cypress EZ-PD CCGx product supporting CC bootloading feature

  - A USB Type-C Electronically Marked Cable Assembly (EMCA) cable

  - A USB Micro-B Cable

  - PC/Laptop installed with the Cypress EZ-PD Configuration Utility

Note: The CY4532 kit installer installs the EZ-PD Configuration Utility by default.

### Using the CY4532 Power Board as a Programmer

- Remove the power supply connected on the DC Jack (J3) of the CY4532 Power Board.

- Detach the CY4532 Main Board pre-mounted on top of the CY4532 Power Board.

Note:

  - To remove the CY4532 Main Board, it is recommended to hold it around the longer edges of the board and lift it up.

  - No specific jumper setting is required on the CY4532 Power Board to use it as a Programmer.

- Connect a USB Type-C EMCA cable between the USB Type-C port (J13) of the CY4532 Power Board and the USB Type-C port on the external hardware.

- Connect the USB Micro-B cable between the USB Micro-B port (J12) of CY4532 Power Board and a USB Type-A port of the PC/laptop.

Note: For the Rev ** of the CY4532 EVK, use USB Type-A to Mini-B cable.

- See Appendix A4

- Connect the 12 V DC power adapter provided with the kit to the CY4532 Power Board’s DC Jack (J3).

- Set the Switch (SW1) on the Power Board to the ‘DC INPUT’ position.

- Invoke the EZ-PD Configuration Utility on the PC/laptop and follow the programming procedure described in the User

## After    first   FW update   we get following message box

## The second attempt failed because   power board does not recognize cypd3175 device

## Questions:

### Q1:  what steps we  need  to  do  in order  require  ability  FW  update  via power board  ?

### Q2:  does bootloader we use   support   "bootloading via cc lines" feature if not please provide or we need implement one?

Appendix

# Appendix A bootloader in PSoC IDE

## Appendix A1

## Appendix A2

## Appendix A3

## Appendix A4

## Appendix A5

## Appendix A6